🟢 SMART INCUBATOR — Hardware Details

1️⃣ Core Controller

- ESP32 Development Board (Dual-Core MCU, Wi-Fi + Bluetooth)

- Power Requirement: 5V DC

- Logic Level: 3.3V

2️⃣ Display Module

- 16×4 I²C LCD (PCF8574 I/O Expander)

- I²C Address: 0x27

- Connections:

  - SDA → GPIO 21

  - SCL → GPIO 22

3️⃣ Temperature & Humidity Sensors

A) Internal Chamber Sensor (Primary Control Sensor)

- DHT22 – Inside Incubator

- Measures: Temperature + Humidity

- Pin:

  - Data → GPIO 4

B) External Ambient Sensor (Diagnostic + Safety Logic)

- DHT11 – Outside Incubator

- Measures: Room Temperature + Humidity

- Pin:

  - Data → GPIO 17

4️⃣ User Input Controls

A) Rotary Encoder (Navigation + Selection)

- CLK → GPIO 32

- DT → GPIO 33

- SW (Push Button) → GPIO 25

B) Physical Buttons

- Back Button → GPIO 16

- Exit Button → GPIO 15

5️⃣ Relay Output Controls (Active-LOW Relay Board)

Used to control incubation hardware components:

| Function | Device Controlled | GPIO Pin |
| --- | --- | --- |
| Heater Relay | Heating Element (Lamp/Heater Coil) | GPIO 27 |
| Humidifier Relay | Water Pump / Ultrasonic Mist | GPIO 26 |
| Fan Relay | Ventilation / Cooling Fan | GPIO 14 |
| Motor Relay | Egg Turning Motor | GPIO 12 |

Relays powered via external 5V supply (ESP32 common GND)

6️⃣ Motor Position Safety Sensors (IR Sensors)

Used for Egg-Turn Position Feedback

- Left IR Sensor → GPIO 35

- Right IR Sensor → GPIO 34

If feedback not detected within 8 seconds → Auto-Stop Motor Fail-Safe

7️⃣ Real-Time Clock Module

- RTC DS3231

- Interface: I²C

- Shared lines with LCD:

  - SDA → GPIO 21

  - SCL → GPIO 22

8️⃣ SD Card Storage (Phase-B Features)

Used for:

- Profiles (JSON)

- Logs

- Recovery Data

SPI Interface Pins

| Function | GPIO |
| --- | --- |
| CS | GPIO 5 |
| SCK | GPIO 18 |
| MOSI | GPIO 23 |
| MISO | GPIO 19 |

9️⃣ Power & Electrical Considerations

- ESP32 Powered via: 5V USB / 5V Regulated Supply

- Relay Board Powered via: External 5V Supply

- Common Ground Required between:

  - ESP32

  - Relay Board

  - RTC / LCD / Sensors

🟢 1️⃣ CURRENT REQUIRED LIBRARIES (Phase-A — Offline Core Firmware)

Core MCU & Hardware Drivers

- Arduino.h

- Wire.h (I²C bus)

LCD Display

- LiquidCrystal_PCF8574.h

Sensors

- DHT.h (DHT22 + DHT11 support)

RTC (Real-Time Clock)

- RTClib.h (DS3231 support)

Internal Logic Helpers (Optional but Recommended)

- EEPROM.h (store fallback values / last valid state)

- Preferences.h (NVS storage — ESP32 safe config memory)

👉 ये libraries Phase-A में 100% उपयोग होंगी
(Offline control + LCD + RTC + Sensors + Decision-Engine Core)

🟡 2️⃣ FUTURE LIBRARIES (Phase-B — SD Card + Profiles + Logging)

SD Card & File Handling

- SPI.h

- SD.h

- FS.h

JSON Handling (Profiles / Settings / Logs)

- ArduinoJson.h (Profile JSON load/store)

Data Recovery & Diagnostics

- CRC32.h (optional — profile integrity check)

- TimeLib.h (optional timestamp helpers)

👉 ये libraries SD-Based features के लिए होंगी:

- Profile loading from SD

- Profile cloning to internal memory

- Log files

- Recovery & resume system

🔵 3️⃣ FUTURE LIBRARIES (Phase-C — Web UI + API + Wi-Fi Mode)

Wi-Fi Access Point / Offline UI

- WiFi.h

- WiFiClient.h

Web Server

Recommended (fast + async):

- ESPAsyncWebServer.h

- AsyncTCP.h

Fallback option (simple server):

- WebServer.h

WebUI Data Encoding

- ArduinoJson.h (shared with SD phase — reused)

Real-time Web Updates (Optional)

- WebSocketsServer.h

🟣 4️⃣ ADVANCED / OPTIONAL SMART-INCUBATOR FEATURES (Future Enhancements)

OTA Firmware Update

- ArduinoOTA.h

MQTT / IoT Expansion

- PubSubClient.h

Graph / Dashboard Logging

- LittleFS.h (internal flash logs instead of SD — optional)

Safety / Reliability Tools

- esp_task_wdt.h (Watchdog hard fail-safe)

- Ticker.h (Timed loops without delay)

🧩 📌 SHORT CHECKLIST — LIBRARY GROUP SUMMARY

✔ Phase-A ( Required Now )

- Wire.h

- LiquidCrystal_PCF8574.h

- DHT.h

- RTClib.h

- EEPROM.h / Preferences.h

✔ Phase-B ( SD + Profiles )

- SPI.h

- SD.h

- FS.h

- ArduinoJson.h

✔ Phase-C ( Web UI )

- WiFi.h

- ESPAsyncWebServer.h

- AsyncTCP.h

✔ Optional / Pro Features

- WebSocketsServer.h

- ArduinoOTA.h

- PubSubClient.h

- esp_task_wdt.h

👍 यह सेक्शन Word-Document के लिए तैयार है

अगर चाहो तो मैं इसे table format या bullet-structured spec sheet style में भी बदल सकता हूँ।

❓ अगला हिस्सा किसका बनाना है?

चुनो:

1️⃣ Firmware Rules & Decision-Engine (Short Technical Spec)
2️⃣ Settings Menu (13 items — documentation format)
3️⃣ LCD Slide Structure Documentation
4️⃣ Profile & Cooling-Logic Explanation

जो बोलोगे — उसी format में तैयार कर दूँगा 👌

🧩 Summary Table — GPIO Pin Map

| Component | Function | GPIO |
| --- | --- | --- |
| LCD I²C SDA | Display | 21 |
| LCD I²C SCL | Display | 22 |
| DHT22 | Internal Sensor | 4 |
| DHT11 | External Sensor | 17 |
| Encoder CLK | Navigation | 32 |
| Encoder DT | Navigation | 33 |
| Encoder SW | Select Button | 25 |
| Back Button | UI Control | 16 |
| Exit Button | UI Control | 15 |
| Heater Relay | Output | 27 |
| Humidifier Relay | Output | 26 |
| Fan Relay | Output | 14 |
| Motor Relay | Output | 12 |
| IR Left | Motor Feedback | 35 |
| IR Right | Motor Feedback | 34 |
| SD CS | Storage | 5 |
| SD SCK | Storage | 18 |
| SD MOSI | Storage | 23 |
| SD MISO | Storage | 19 |

👍 Ready for Document Use

यह section बिल्कुल industrial-spec doc style में लिखा गया है
तुम इसे सीधे Word फ़ाइल में डाल सकते हो।

❓ अगला सेक्शन किसका बनाना है?

Option चुनो:

1️⃣ Firmware philosophy / decision-engine rules (short format)
2️⃣ Slide UI structure description
3️⃣ Settings menu explanation (13 items)
4️⃣ Profile + incubation control workflow

जो बोलो — उसी format में तैयार कर देता हूँ 👌